ASSESSMENT MATERIAL

Learning Unit 2

CONNECTORS AND ADHESIVES

UPWOOD

Up-skilling construction workers in wood construction methods for energy-efficient buildings

UPWOOD-PUU

Rakennustyöläisten ammattitaito energiatehokkaiden rakennusten puurakentamisenmenetelmissä

UPWOOD

Rakennustyöläisten ammattitaito energiatehokkaiden rakennusten puurakentamisenmenetelmissä

UPWOOD-PUU

Rakennustyöläisten ammattitaito energiatehokkaiden rakennusten puurakentamisenmenetelmissä

UPWOOD

Rakennustyöläisten ammattitaito energiatehokkaiden rakennusten puurakentamisenmenetelmissä

UPWOOD-PUU

Rakennustyöläisten ammattitaito energiatehokkaiden rakennusten puurakentamisenmenetelmissä

UPWOOD

Rakennustyöläisten ammattitaito energiatehokkaiden rakennusten puurakentamisenmenetelmissä

UPWOOD-PUU

Rakennustyöläisten ammattitaito energiatehokkaiden rakennusten puurakentamisenmenetelmissä

Table of contents

1.	Frequently asked questions	2

2.	Multiple choice questions	3

3.	Case studies	3

3.1	Case study 1	3

3.2	Case study 2	3

Frequently asked questions

Question: What does wood welding mean?

Answer: Welding wood is a method of replacing gluing.

Question: What adhesives are available for different structures or structural conditions?

Answer:

- Weatherproof adhesives: Adhesives must be more durable than wood in all conditions.

- Moisture-resistant adhesives: Adhesives are suitable for indoor use where the relative humidity may be quite high.

- Adhesives for indoor use: Adhesives do not withstand water soaking and will last for a limited time in humid conditions.

Multiple choice questions

- For gluing load-bearing structures, what type of adhesive is most suitable from given options below:

  - natural glue

  - resorcinol adhesives

  - epoxy adhesives

- Phenol adhesives are

  - used in the plywood industry as a high frequency adhesive.

  - suitable for indoor use as it do not withstand water soaking.

  - classified into 4 main types, two- or one-component reaction adhesives, solvent adhesives and aqueous dispersion adhesives.

- Cohesion means

  - the internal strength of the adhesive and the adhesive joint.

  - the process where the adhesive penetrates to pores of the wood.

Case studies

  - Case study 1

Name at least 5 different joining techniques. Describe both pros and cons of them. Do you notice some joining techniques used in your room’s window frames or furniture?

  - Case study 2

Wood welding is alternative method where two pieces of wood can be joined without using glue. Describe the method of wood welding in words.